Supporting Information for

Environmental Impacts of Lithium Supply Chains from Australia to China

Yan Feng1, Danyi Feng2, , Wen Li3, Qian Zhang4, Wei-Qiang Chen1,5,6, and Peng Wang1,5,6,

- Key Lab of Urban Environment and Health, Institute of Urban Environment, Chinese Academy of Sciences, Xiamen, Fujian 361021, China

- University of Wisconsin-Madison, Department of Civil and Environmental Engineering, 1415 Engineering Drive, Madison, WI, 53706, USA

- Department of Mechanical Engineering, University of Melbourne, Melbourne, VIC 3010, Australia

- Robert M. Buchan Department of Mining, Queen’s University, Kingston, K7L3N6, Canada

- Ganjiang Innovation Academy, Chinese Academy of Sciences, Ganzhou 341000, China

- University of Chinese Academy of Sciences, Beijing 100049, China

Contents

1.	System boundary	3

2.	Contribution of impact categories	4

3.	Life cycle inventory datasets	5

4.	Electricity generation mix	12

5.	Provincial Li plants shipping information	13

6.	LCA results	14

- System boundary

Figure S1 Flows and processes of lithium carbonate (LCE) production included in this life-cycle assessment model (1 t battery-grade LCE produced).

- Contribution of impact categories

Figure S2 Contribution of environmental impact categories based on normalized results.

- Production capacity of Australian mines and China processing plants

Tabel S1 Comprehensive information on lithium (Li) extraction by Australian mines and processing plants in China.

| Production routes | Raw ore grade (Li2O %) | Li extraction rate | production capacity (t/years) | Concentrate demand (t) | Source |
| --- | --- | --- | --- | --- | --- |
| Gre-SN | 1.6 | Beneficiation: 75% Refining: 82% | Concentrate: 1340000 LCE: 20000 | 164466 | http://www.tianqilithium.com/Upload/File/202304/20230429011029_0862.pdf |
| Gre-QZ | 1.6 | Beneficiation: 75% Refining: 85% | Concentrate: 1340000 LCE: 3000 | 30142 | http://www.tianqilithium.com/Upload/File/202304/20230429011029_0862.pdf |
| Cat-YA | 1.2 | Beneficiation: 52% Refining: 93% | Concentrate: 131000 LCE: 20000 | 152556 | https://www.datocms-assets.com/53992/1699424132-ake-fy23-sustainability-report-2023.pdf |
| Mar-XY | 1.37 | Beneficiation: 79% Refining: 95% | Concentrate: 600000 LCE: 15000 | 106200 | https://s3.ap-southeast-2.amazonaws.com/assets.mineralresources.com.au/app/uploads/2022/10/13200621/MinRes-2022-Annual-Report-20220915.pdf |
| Wod-XY | 1.17 | Beneficiation: 68% Refining: 95% | Concentrate: 750000 LCE: 15000 | 106200 | https://s3.ap-southeast-2.amazonaws.com/assets.mineralresources.com.au/app/uploads/2022/10/13200621/MinRes-2022-Annual-Report-20220915.pdf |
| Pil-HM | 1.1 | Beneficiation: 73% Refining: 82% | Concentrate: 377902 LCE: 6000 | 48503 | https://1pls.irmau.com/site/pdf/84195c02-1519-47cb-8e0b-8ef4322bfa5b/2022-Annual-Report.pdf |

- Life cycle inventory datasets

The process chains begin with a conventional open-pit mining process. Due to the lack of plant-specific inventory data (e.g., energy supply, explosives), the mining inventory is sourced from the EcoInvent database (Spodumene {AU}| spodumene production | Cut-off, S).

Table S2 presents material and energy inputs to produce concentrated spodumene from western Australia mines. The inventory data for Greenbushes, Marion, and Wodgina are based on data published by Kelly et al. and are converted according to the ore grade. For Cattlin, the beneficiation inventory data comes from Allkem' s “Sustainability Report 2022”, while Pilgangoora's data is sourced from Pilbara's “Annual Report 2022”.

The LCI details all inputs of material, energy, and water required to produce one metric ton of LCE. In this study, the LCI data for LCE produced in China was sourced from the project’s Environmental Impact Assessment Report (EIA). The upstream processes, including resource extraction, raw material production, energy generation, and transportation, are based on Ecoinvent 3.9. All LCIs are listed in Tables 3, described as follows.

Tabel S2 Life cycle inventory of spodumene concentrate per ton produced by Australian mines.

| Input /t Concentrate | Greenbushes | Cattlin | Marion | Wodgina | Pilgangoora |
| --- | --- | --- | --- | --- | --- |
| Materials input |  |  |  |  |  |
| Spodumene ore [t] | 5 | 9.044 | 5.5 | 7.6 | 6.75 |
| Fresh water [t] | 3.333 | 5.259 | 3.67 | 5.1 | 4.5 |
| Othera [t] | 0.017 | - | 0.018 | 0.025 | - |
| Energy input |  |  |  |  |  |
| Diesel [MJ] | 5000 | 2654.7 | 5500 | 7600 | 2271 |
| Electricity [kWh] | - | 244.69 | - | - | 192.25 |

a Includes sodium carbonate and a dispersant.

Tabel S3 Life cycle inventory of Li2CO3 per ton produced by Factories in China.

| Input /t LCE | Gre-SN | Gre-QZ | Cat-YA | Mar-XY | Wod-XY | Pil-HM |
| --- | --- | --- | --- | --- | --- | --- |
| Materials input |  |  |  |  |  |  |
| Concentrate | 8.223 | 10.047 | 7.628 | 10.838 | 10.874 | 8.084 |
| Sulfuric acid | 2.68 | 1.903 | 1.115 | 2.501 | 2.501 | 2.082 |
| Soda ash | 2.35 | - | - | 0.023 | 0.023 | 1.572 |
| Sodium hydroxide | 0.48 | 2.076 | 0.659 | 0.144 | 0.144 | 0.005 |
| Calcium carbonate | 0.7 | 1.278 | 0.274 | 0.665 | 0.665 | 0.539 |
| Limestone | - | 0.010 |  |  | - | 0.539 |
| Carbon dioxide | 0.3 | 0.646 | 0.176 | 0.700 | 0.700 | - |
| Hydrochloric acid | 0.17 | - | - | - | - | - |
| Oxygen, liquid | 0.035 | - | - | - | - | - |
| Fresh water | 29.68 | 20.473 | 18.405 | 0.496 | 0.496 | 18.6 |
| Energy input |  |  |  |  |  |  |
| Electricity [kWh] | 2900 | - | 71.470 | - | - | 1900 |
| Natural gas [m3] | 1414.5 | 1377 | - | - | - | 0.085 |
| Steam | - | 0.999 | 7.004 | - | - | 9.340 |
| Coal |  | - |  | 1.929 | 1.929 | - |

Data source:

Gre-SN: https://maipdf.com/est/d17423523236@pdf

Gre-QZ: https://maipdf.com/est/d15527523549@pdf

Cat-YA: https://maipdf.com/est/d12526527214@pdf

Mar-XY: https://maipdf.com/est/d14089527598@pdf

Wod-XY: https://maipdf.com/est/d14089527598@pdf

Pil-HM: https://maipdf.com/est/d17389765945@pdf

- Electricity generation mix

Characterizing the electrical grids that will supply power to electricity-consuming processes remains crucial. Detailed information for both Australia and China is furnished in Table S.4-5. The most recent data available for Australian electricity from the International Energy Agency website is for 2022. For China, the latest information from the same source is for 2021.

Tabel S4 Electricity generation mix of electricity consumption for Li production in Australia.

|  |  |  |  | Electricity mix |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Grid | Coal | Oil | Natural gas | Biofuels | Hydro | Solar PV | Wind |
| Australia | 49.2% | 1.7% | 18.1% | 1.2% | 6.3% | 12.8% | 10.7% |

Tabel S5 Electricity generation mix of electricity consumption for Li production in China.

|  |  |  |  | Electricity mix |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Grid | Coal | Nuclear | Hydro | Wind | Natural gas | Biofuels | Solar PV |
| China | 63.0% | 4.7% | 15.6% | 7.6% | 3.2% | 2.0% | 3.9% |

- Provincial Li plants shipping information

For the assumed Australian mining facility, we assume that the spodumene concentrate is transported to the port by truck within inland Western Australia. Then we provide the assumed value for the transportation of spodumene concentrate to China's port by ship and its further movement by truck to the final processing location for refinement into Li2CO3.

Tabel S6 Transportation of spodumene concentrate from Australia to China (units: km)

| Project | Distance | Transportation | Location |
| --- | --- | --- | --- |
| Greenbushes | 80.5 | Truck | Greenbushes—Bunbury |
|  | 4980 | Ship | Bunbury—Haian (Jiangsu) |
|  | 1435 | Truck | Haian (Jiangsu)—Suining (Sichuan) |
| Mt Marion | 347 | Truck | Mt Marion—Esperance |
|  | 5029 | Ship | Esperance—Xiamen (Fujian) |
|  | 484 | Truck | Xiamen(Fujian)—Fengcheng（Jiangxi） |
| Mt Cattlin | 196 | Truck | Mt Cattlin—Esperance |
|  | 4709 | Ship | Esperance—Modern Terminals |
|  | 1358 | Truck | Modern Terminals—Danling (Sichuan) |
| Wodgina | 116 | Truck | Mt Wodgina—Port of Hedland |
|  | 3488 | Ship | Port of Hedland —Xiamen (Fujian) |
|  | 484 | Truck | Xiamen(Fujian)—Fengcheng（Jiangxi） |
| Pilgangoora | 109 | Truck | Mt Pilgangoora——Port of Hedland |
|  | 3651 | Ship | Port of Hedland——Haian (Jiangsu) |
|  | 1149 | Truck | Haian (Jiangsu)——Haimen (Jiangsu) |

- LCA results

Tabel S7 Cradle-to-gate life cycle results per ton of concentrate ore for the Australian mines

| Indicators | Units | Greenbushes | Cattlin | Marion | Wodgina | Pilgangoora |
| --- | --- | --- | --- | --- | --- | --- |
| GWP | kg CO2 eq | 7.50E+02 | 8.64E+02 | 8.25E+02 | 1.14E+03 | 6.80E+02 |
| ODP | kgCFC11 eq | 6.65E-04 | 6.57E-04 | 7.31E-04 | 1.01E-03 | 5.19E-04 |
| PMFP | kg PM2.5 eq | 3.16E+00 | 3.35E+00 | 3.48E+00 | 4.81E+00 | 2.62E+00 |
| TAP | kg SO2 eq | 4.86E+00 | 4.83E+00 | 5.34E+00 | 7.38E+00 | 3.83E+00 |
| FEP | kg P eq | 3.93E-01 | 7.77E-01 | 4.32E-01 | 5.97E-01 | 5.93E-01 |
| FETP | kg 1,4-DCB | 2.80E+01 | 3.48E+01 | 3.08E+01 | 4.26E+01 | 2.73E+01 |
| HTPc | kg 1,4-DCB | 4.38E+01 | 6.48E+01 | 4.81E+01 | 6.65E+01 | 5.03E+01 |
| FFP | kg oil eq | 2.12E+02 | 2.28E+02 | 2.33E+02 | 3.22E+02 | 1.81E+02 |
| WCP | m3 | 6.83E+00 | 1.06E+01 | 7.51E+00 | 1.04E+01 | 8.52E+00 |

Figure S3 Percentage contributions of various energy and material inputs to the environmental and human health impacts based on the Australian concentrate process.

| Tabel S8 Cradle-to-gate life cycle results per ton of Li2CO3 for the production routes. | Pil-HM | 1.45E+04 | 5.72E-03 | 8.17E+01 | 1.12E+02 | 7.58E+00 | 8.98E+02 | 9.34E+02 | 3.70E+03 | 1.58E+02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Wod-XY | 1.52E+04 | 1.18E-02 | 6.21E+01 | 1.05E+02 | 8.54E+00 | 1.13E+03 | 9.83E+02 | 4.89E+03 | 1.66E+02 |
|  | Mar-XY | 1.17E+04 | 8.75E-03 | 4.75E+01 | 8.28E+01 | 6.73E+00 | 9.99E+02 | 7.81E+02 | 3.91E+03 | 1.34E+02 |
|  | Cat-YA | 1.05E+04 | 6.60E-03 | 3.46E+01 | 5.73E+01 | 7.26E+00 | 6.02E+02 | 6.68E+02 | 2.82E+03 | 1.43E+02 |
|  | Gre-QZ | 1.24E+04 | 1.05E-02 | 4.46E+01 | 7.93E+01 | 6.39E+00 | 9.43E+02 | 7.97E+02 | 4.53E+03 | 1.97E+02 |
|  | Gre-SN | 1.64E+04 | 7.96E-03 | 1.15E+02 | 1.53E+02 | 7.00E+00 | 1.18E+03 | 1.15E+03 | 5.23E+03 | 2.01E+02 |
|  | Units | kg CO2 eq | kg CFC11 eq | kg PM2.5 eq | kg SO2 eq | kg P eq | kg 1,4-DCB | kg 1,4-DCB | kg oil eq | m3 |
|  | Indicators | GWP | ODP | PMFP | TAP | FEP | FETP | HTPc | FFP | WCP |

Corresponding author.

E-mail address: dfeng26@wisc.edu (D.Y. Feng), pwang@iue.ac.cn (P. Wang).